Post-Brexit: Implications on Irish Freight, Transport, and Logistics Sector

Technical Report

30 June 2020

- Contents

1	Introduction	3

2	Model Scope	4

3	Methodology	5

3.1	Data Collection	5

3.1.1	Trade volumes handled by Irish ports	5

3.1.2	Volume of Ro/Ro freight units (i.e. trucks) handled by Irish ports	9

3.1.3	Ro/Ro vessels’ sailing schedule and capacity	10

3.2	Understand Brexit risks and conceptual modelling	11

3.2.1	Expert interviews, focus groups and field work	11

3.2.2	Conceptual models for the trade flow and border interventions	11

3.2.3	Model input parameters and performance indicators	15

3.3	Simulation-based scenario mapping model	16

Appendix	18

List of Input Parameters	18

List of Performance Indicators	25

# Introduction

Irish maritime transport is built upon a strategic route-to-market infrastructure, connecting the Republic of Ireland (ROI) to the majority of its international markets. According to the Irish Maritime Development Office (IMDO), more than 90% of Ireland's global trade volumes move through its seaports (IMDO 2019). The total amount of goods which passes through Irish ports using Roll on-Roll off (Ro/Ro), is around 16 million tonnes. More than 86% of this, around 13.8 million tonnes, is exported to the UK, while the remaining 16% uses direct routes to continental Europe. Apart from their close trade relationship, Ireland has historically relied on the UK road and ports network, known as the UK Land-bridge, as their primary route-to-market to mainland Europe. The UK Land-bridge is favoured by Irish and EU traders as it offers a border-free route. There is a high-frequency of short-sea crossings, with significantly faster transit times, than alternative direct routes to the EU. The uncertainty for the future of this route has drawn scrutiny surrounding it. In fact, the core mission for the Irish government is to mitigate the risk of trade disruptions caused by Brexit. They also want to maintain the close trade between the UK, Ireland and wider Europe, whilst minimizing the regulatory burden for all goods being imported and exported: 1. To the UK for direct UK trade. 2. To continental Europe via UK land-bridge or direct routes.

Therefore, this study aims to firstly analyse and then quantify the potential disruptions of Brexit on the trade flow between Ireland, the UK (referred hereafter as GB), and the EU from an operational perspective. The main objective of this study is:

“To develop a simulation-based scenario mapping tool to examine and analyse the Brexit implication on Irish freight, transport and logistics industry.”

Based on this central objective, these specific objectives have been designed:

# Model Scope

The focus of this model is the RO/RO traffic between Ireland, GB and continental Europe. This traffic is either through Land-bridge or direct route. RO/RO traffic from Ireland and Northern Ireland moves through four corridors: Northern, Eastern, Southern, and Continental. While the Northern corridor links Northern Ireland (NI) with GB through Scotland, the Central and Southern corridors connect Ireland with GB and the EU through the Land-bridge. On the other hand, the direct continental service connects Ireland directly to Continental Europe through the ports of France, The Netherlands, and Belgium. Therefore, the project focuses on the investigation of Brexit implications on the RO/RO transportation performance within these three main corridors:

- East/West Maritime Corridor: This includes the central and Southern corridors through Dublin – Holyhead, Dublin – Liverpool, Dublin – Heysham, Rosslare – Fishguard, and Rosslare-Pembroke maritime routes.

- Land-bridge Route: This models the transportation links between Ireland and the EU through, 1) East/West maritime corridor. 2) Transit routes from Holyhead, Liverpool, Heysham, Fishguard, and Pembroke ports, at the Western side of the GB. With Dover port, at the Eastern side. 3) The maritime route Dover – Calais in France to mainland Europe.

- Direct Route to the Continental Europe: This model links Ireland directly with the EU through three maritime routes: 1) Dublin & Rosslare – Cherbourg at France. 2) Dublin – Rotterdam at The Netherlands. 3) Dublin – Zeebrugge at Belgium.

Figure 1. Maritime shipping routes between Ireland, GB, and the EU.

# Methodology

The methodology of the study is based on various data sources and modelling activities. This firstly includes data collection from literature, data banks, and expert interviews. Then secondly, conceptual modelling, simulation model development, and sensitivity analysis of scenario outcomes.

## Data Collection

The data collection was taken from various data sources such as the CSO (Central Statistics Office) of Ireland, EuroStat, the Irish Maritime Development Office publications (IMDO), the Department of Agricultural, Food and the Marine (DAFM) reports, Dublin and Rosslare ports websites, shipping company websites, and other governmental publications.

### Trade volumes handled by Irish ports

| The data shows that Ireland’s trade volume of both imports and exports in 2018 worldwide was recorded at 55 million tonnes. 67% of this volume is imported from all regions. The trade volume between Ireland and the UK (i.e. Great Britain & Northern Ireland) is reported to be 40% of the total tonnage, handled by region of trade. While the trade with the remaining EU countries accounts for 34.1%. Five categories of traffic are employed to carry the traded goods to and from Ireland. These are Liquid Bulk, Dry Bulk, Lift-on/Lift-off traffic, Roll-on/Roll-off traffic, and Break bulk. RO/RO is the most popular category engaged between Ireland and GB. It accounts for approximately 63% of the Irish trade within the UK. On the other hand, Dry Bulk and LO/LO categories dominate the traffic between Ireland and the remaining EU countries with approximately 75% of the trade. While RO/RO presents only 12% of the trade with the other EU countries. | Table 1. Tonnage of goods handled at Irish Ports (2018). Category of goods; Volume ('000 tonnes). Goods received. Liquid bulk; 9,806. Dry bulk; 13,555. Lift-on/lift-off; 4,138. Roll-on/roll-off; 8,718. Break bulk and other goods; 871. Total; 37,088. Goods forwarded. Liquid bulk; 2,364. Dry bulk; 4,199. Lift-on/lift-off; 3,491. Roll-on/roll-off; 7,319. Break bulk and other goods; 624. Total; 17,998. Total Goods. Liquid bulk; 12,171. Dry bulk; 17,754. Lift-on/lift-off; 7,629. Roll-on/roll-off; 16,037. Break bulk and other goods; 1,496. Total; 55,086. Source: CSO.. |
| --- | --- |

#### RO/RO trade volumes of all product (Agri and non-Agri) within the three corridors

After investigation into CSO databases and statistical resources, no published figures for the RO/RO traffic volumes which flow between the identified maritime routes in the three corridors, could be found. Therefore, data analysis was based on assumptions, along with consultations from importers, exporters, port officials, and trade associations. These are explained below:

- Ro/Ro trade volumes between Ireland and GB through the East/West Maritime Corridor:  According to CSO, the trade between the East/West Corridor are either RO/RO traffic or Liquid bulk. This includes products such as crude oil, oil products, liquefied gas. Therefore, when the oil products volumes are removed, it can be assumed that all products traded within the routes in the East/West Corridor are RO/RO traffic.

- Ro/Ro trade volumes between Ireland and EU through Land-bridge:  According to the IMDO report in 2017, 16% of the RO/RO traffic between Ireland and GB use the Land-bridge to transport goods to and from European ports. Based on these resources, the RO/RO traffic between Ireland and the EU, through Land-bridge, is calculated and illustrated in Table 2.

- Ro/Ro trade volumes between Ireland and EU through direct routes: The consulted experts also indicate that direct routes to continental EU mainly links Ireland with France, The Netherlands, and Belgium through four maritime routes including Dublin–Cherbourg, Rosslare–Cherbourg, Dublin–Rotterdam, and Dublin–Zeebrugge. The CSO published the accumulated figures of the RO/RO traffic between Ireland the other EU countries showing 1.3 million tonnes and 835,000 tonnes imported and exported from Irish ports respectively. However, the CSO figures did not clarify the proportion of volumes within each of the four direct maritime routes. Therefore, the Eurostat database was used to reveal these figures as illustrated in Table 2. A narrow gap can be seen between the Eurostat and CSO accumulated figures, ensuring the accuracy of the research and the identified figures.

#### RO/RO trade volumes of Agri products within the three corridors

The project provides particular attention to the impact the introduction of new non-tariff barriers would have. This in particular at Irish and the GB ports following the transition period. Non-tariff barriers can include sanitary and phytosanitary (SPS) measures, conformity assessment, pre-shipment inspections related to rule of origins. These would be applied to live animals and animal products, as well as plant products entering the EU single market. The DAFM specified 7 product categories which would incur these checks including:

- Animals & Animal Product.

- Plants & Plant Products.

- Timber & Timber Products.

- Fishery Products.

- Animal Feed.

- Fertilizers.

- Food of non-animal origin subject to increased pesticide residue control.

Table 2. RO/RO trade volume within every studied maritime route.

| Maritime Route |  | Volume (tonnes) |
| --- | --- | --- |
| Goods received |  |  |
| East/West Maritime Corridor | Heysham – Dublin | 356,000 |
|  | Holyhead – Dublin | 3,470,000 |
|  | Liverpool – Dublin | 2,982,000 |
|  | Fishguard – Rosslare | 247,000 |
|  | Pembroke – Rosslare | 481,000 |
| Land-bridge route | Volumes between Ireland and EU (Landbridge) | 1,205,000 |
| Direct route | Cherbourg – Dublin | 41,000 |
|  | Cherbourg – Rosslare | 182,000 |
|  | Rotterdam – Dublin | 417,000 |
|  | Zeebrugge – Dublin | 588,000 |
| Goods Forwarded |  |  |
| East/West Maritime Corridor | Dublin – Heysham | 273,000 |
|  | Dublin – Holyhead | 2,846,000 |
|  | Dublin – Liverpool | 2,496,000 |
|  | Rosslare – Fishguard | 330,000 |
|  | Rosslare – Pembroke | 667,000 |
| Land-bridge route | Volumes between Ireland and EU (Landbridge) | 1,057,000 |
| Direct route | Dublin – Cherbourg | 56,000 |
|  | Rosslare – Cherbourg | 202,000 |
|  | Dublin – Rotterdam | 256,000 |
|  | Dublin – Zeebrugge | 287,000 |

Source: Adapted from the CSO & EuroStat.

According to the CSO in conjunction with the project’s experts, 14 product divisions can be assumed, under each of the 7 categories presented in the Agri products which flow between Ireland, GB and the EU. These would be subject to non-tariff barriers checks after the transition period, see Table 3.

Table 3. Agri products division and volumes through Ro/Ro traffic.

| Product Category | Product Division | Trade Volume with GB - 2018 |  | Trade Volume with Other-EU – 2018 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Export Volume (tonnes) | Import Volume (tonnes) | Export Volume (tonnes) | Import Volume (tonnes) |
| Animal & Animal Products | 00 – Live Animal | 9,277 | 1,973 | 20,206 | 542 |
|  | 01 – Meat & Meat Preparation | 454,859 | 122,903 | 319,510 | 143,001 |
|  | 02 – Dairy Products | 16,872 | 46,125 | 90,088 | 19,244 |
|  | 03 – Crude Animal & vegetable material | 16,872 | 46,125 | 90,088 | 19,244 |
|  | 41 - Animal oils & fats | 20,718 | 2,941 | 69,503 | 2,764 |
| Plant & Plant Product | 05 – Vegetable & Fruit | 140,129 | 365,938 | 8,261 | 471,542 |
|  | 43 - Animal or vegetable materials | 1,417 | 20,101 | 7 | 6,940 |
| Timber & Timber Products | 24 – Cork & Wood | 373,358 | 234,173 | 2,537 | 184,748 |
|  | 63 – Cork & wood manufactures (excl. furniture) | 360,771 | 67,845 | 131,080 | 137,901 |
| Fishery Product | 03 – Fish & molluscs Preparation | 16,872 | 46,125 | 90,088 | 19,244 |
| Animal Feed | 08 – Feeding Stuff for Animal | 181,664 | 397,873 | 81,645 | 946,351 |
|  | 09 - Hides, skins & fur-skins | 101,479 | 153,388 | 140,363 | 115,581 |
| Fertilizer | 27 – Crude fertilizer & minerals | 135,831 | 388,552 | 50,605 | 921,773 |
|  | 56 - Fertilizers | 3,424 | 142,571 | 474 | 985,999 |
| RO/RO Agri product trade volume between Ireland and GB* |  | 1,540,176 | 1,710,772 |  |  |
| RO/RO Agri product trade volume between Ireland and EU (Land-bridge) ** |  | 293,367 | 325,862 |  |  |
| Total Agri product trade volume between Ireland and EU (Direct Route) |  |  |  | 1,094,455 | 3,974,874 |
| RO/RO Agri product trade volume between Ireland and EU (Direct Route) *** |  |  |  | 120,390 | 516,734 |
| Source: The CSO. *All trade through the East/West maritime corridor to GB ports are assumed to be transported using Ro/Ro traffic mode. **According to IMDO report, all trade between Ireland and GB is shipped through RO/RO traffic where 16% of this volume are directed to EU through Land-bridge. *** According to CSO, the percentages of both inbound and outbound RO/RO traffic between Ireland and the EU are 11% and 13% respectively of the total trade volume. |  |  |  |  |  |

### Volume of Ro/Ro freight units (i.e. trucks) handled by Irish ports

Identifying the number of vehicles which flow within the three studied corridors (i.e. East/West maritime corridor, Land-bridge, and direct route to the continent), is important in understanding the operations at the Irish and GB ports. The CSO provides figures on the RO/RO freight vehicles which cross the Irish sea between Ireland and the GB. However, there is no breakdown in these figures within the individual maritime routes which link the two countries. Similarly, it is also not clear what number of vehicles move to mainland Europe, through the land-bridge or direct route. Pinpointing these numbers is important in understanding the dynamics of the transportation network within the highlighted corridors. Also, in investigating the impact the introduction of non-tariff barriers to truck waiting time is essential. According to an IMDO report in 2017, the products trade volumes, expressed by tonnes, can be converted into number of vehicles by using the Average Unit Weight metric (AUW). AUW is estimated in the IMDO report to be 21 tonnes per truck for the RO/RO Heavy Good Vehicles (HGV), see Table 4.

Table 4. Number of RO/RO freight trucks within every studied maritime route.

| Maritime Route |  | Products Volume (tonnes) | Number of Freight Units AUW (21 tonnes/truck) |
| --- | --- | --- | --- |
| Goods Received |  |  |  |
| East/West Maritime Corridor | Heysham – Dublin | 356,000 | 17,000 |
|  | Holyhead – Dublin | 3,470,000 | 165,238 |
|  | Liverpool – Dublin | 2,982,000 | 142,000 |
|  | Fishguard – Rosslare | 247,000 | 11,762 |
|  | Pembroke – Rosslare | 481,000 | 22,905 |
|  | Total RO/RO trade volumes and freight units received from GB | 7,536,000 | 358,905 |
| Direct route | Cherbourg – Dublin | 41,000 | 1,952 |
|  | Cherbourg – Rosslare | 182,000 | 8,667 |
|  | Rotterdam – Dublin | 417,000 | 19,857 |
|  | Zeebrugge – Dublin | 588,000 | 28,000 |
| Goods Forwarded |  |  |  |
| East/West Maritime Corridor | Dublin – Heysham | 273,000 | 13,000 |
|  | Dublin – Holyhead | 2,846,000 | 135,524 |
|  | Dublin – Liverpool | 2,496,000 | 118,857 |
|  | Rosslare – Fishguard | 330,000 | 15,714 |
|  | Rosslare – Pembroke | 667,000 | 31,762 |
|  | Total RO/RO trade volumes and freight units forwarded to GB | 6,6120,000 | 314,857 |
| Direct route | Dublin – Cherbourg | 56,000 | 2,667 |
|  | Rosslare – Cherbourg | 202,000 | 9,619 |
|  | Dublin – Rotterdam | 256,000 | 12,190 |
|  | Dublin – Zeebrugge | 287,000 | 13,667 |

Source: Adapted from the CSO/IMDO.

### Ro/Ro vessels’ sailing schedule and capacity

Irish Ferries, SeaTruck, P&O, and StenaLine provide Ro/Ro vessels service across the East/West Corridor. They call to Holyhead, Liverpool, and Heysham from Dublin, and to Fishguard and Pembroke from Rosslare. These services grew by 4% in 2018 and now account for 45% of all Ireland’s RO/RO traffic, see IMTE 2018. Irish Ferries and StenaLine also provides a direct route to continental Europe by connecting Cherbourg port in France with Dublin and Rosslare ports, respectively.

These direct routes also consist of CLDN ferries which link Dublin to Rotterdam and Zeebrugge ports in The Netherlands and Belgium respectively. Recently, significant capacities have been added to the direct service to the continent with arrival of new operator, Cobelfert. These have begun direct RO/RO services from Dublin to Zeebrugge, and more recently to Rotterdam. Direct continental services to France, Belgium, and The Netherlands has increased by 15% in total. This indicates the domination of these routes. Detailed ferry routes, transit times, and ferry capacities, along with the frequencies in the East/West and Direct continental Corridors for 2018 are presented in Table 5.

Table 5. Data Collected on Ferries and Vessels of the relevant Maritime Routes.

| Maritime Route |  | Shipping company | Weekly Sailing Frequency (both ways) | Transit Time (hours) | Avg Vessel Capacity (Freight Unit) |
| --- | --- | --- | --- | --- | --- |
| East/West Maritime Corridor | Dublin – Holyhead | Irish Ferries | 56 | 3.5 | 209 |
|  | Rosslare – Pembroke |  | 28 | 4 | 122 |
|  | Dublin – Holyhead | StenaLine | 56 | 3.5 | 209 |
|  | Rosslare – Fishguard |  | 28 | 4 | 75 |
|  | Dublin – Liverpool | P&O | 24 | 8 | 123 |
|  | Dublin – Liverpool | SeaTruck | 42 | 8 | 123 |
|  | Dublin – Heysham |  | 12 | 9 | 122 |
| Direct route | Dublin – Rotterdam | CLDN | 6 | 41 | 530 |
|  | Dublin – Zeebrugge |  | 4 | 40 | 530 |
|  | Dublin – Cherbourg | Irish Ferries | 6 | 19 | 170 |
|  | Rosslare – Cherbourg | StenaLine | 6 | 17 | 150 |

Source: Shipping company’s websites.

## Understand Brexit risks and conceptual modelling

### Expert interviews, focus groups and field work

In order to understand the implications of Brexit on transportation links between Ireland, GB, and the EU, a wide range of qualitative techniques have been utilized. These include focus groups, face-to-face interviews, and sites visits. Since the beginning, the project team was in continuous contact with various senior supply chain professionals working in Irish trade companies from both the import and export perspective, along with agri-food manufacturers, logistic companies, and trade associations.

Interviews were conducted with participants who enjoy a rich past in the strategic and operational aspects of supply chain design, and transportation links, between Ireland and GB. Preliminary interviews and focus groups were conducted with a freight forwarder, an agri-food processor, and a logistics company. These were necessary for initial consultation regarding the scope, feasibility, and parameters of the study. Two site visits were made to Dublin and Rosslare port, along with detailed interviews with experienced port officials. This was to gain a clearer picture of operations, transportation activities, and preparation for Brexit. In total, 21 in-depth interviews were conducted. The interview outcomes, together with the literature review, were used to design the tested scenarios and the following conceptual models:

- Overall trade flow of inbound and outbound trucks between Ireland, GB, and the EU.

- Border interventions (custom, SPS, and security) procedures at Irish, GB, and EU ports.

Furthermore, a multi-disciplinary expert panel, consisting of specialists from different management and economic disciplines, was formed. The professionals provided opinions and a more in-depth analysis to the model outcomes. To date, 5 focus groups and 4 in-depth interviews have been conducted with the panel specialists. In these interactions, the model outcomes were presented and discussed from an economic, food safety, retail management, and food manufacturing perspective.

### Conceptual models for the trade flow and border interventions

- Conceptual model of the overall flow for inbound/outbound trucks: Based on the qualitative data analysis of expert interviews and focus group, various conceptual models were developed. These modelled the complete journey of inbound and outbound trucks from their country of origin to their country of destination. Two main entities, i.e. supplier and distributors, were used in the model to mimic the origin and destination points respectively. In the case of outbound flow from Ireland to GB and continental Europe, trucks which originated from an Irish-based supplier, and are heading through different maritime routes, to finish the journey either at GB-based distributor or EU-based distributor, depending on the trucks destination, as shown in Figure 2.

Figure 2. Outbound trucks flow from Ireland to GB or EU (via land-bridge or direct route).

Similarly, in the inbound flow to Ireland, the trucks which originated from either a GB-based supplier or EU-based supplier go through different ports, and finish their journey at an Irish-based distributor, as shown in Figure 3.

Figure 3. Inbound trucks flow from GB or EU (via land-bridge or direct route) to Ireland.

- Conceptual model of border checks on the inbound/outbound flow: Detailed conceptual models of border checks interventions were prepared. This was based on information collected from the Irish state Agencies (i.e. Revenue and the DAFM). Along with qualitative data analysis from expert interviews. According to the state agencies, non-tariff barriers are inevitable at Irish, GB, and EU ports. Export products from Ireland will involve minimal tests or intervention by customs at clearance in Irish ports, and no checking costs will be charged, see Figure 4.

Figure 4. Border interventions on outbound trucks going from Ireland.

On the other hand, import checks comprise of documentary and sealed identity checks (i.e. import declarations and security documents), along with SPS inspections on a specific percentage of the trucks carrying Agri food products. However, for Non-Agri products, non-physical checks will take place before the ferry docks at Irish ports. The process mapping of import checks is depicted in Figure 5. Similar process logic was used to model the border checks procedures at GB and EU ports.

Figure 5. Border interventions on inbound agri and non-agri trucks coming to Ireland.

### Model input parameters and performance indicators

Based on consultations with the experts, together with the review of the literature and the respective publications, a list of input parameters and performance indicators were identified. This allowed the project to map the tested scenarios in the research, see Table 6. The model can be customized to present Brexit scenarios by altering the values of the related input parameters, and by measuring the associated performance indicators, see Section 4. A detailed list of input parameters and performance indicators are presented in Annex 1.

Table 6. Input parameters and performance indicators.

| Model Input Categories |  |
| --- | --- |
| RO/RO trade volumes between Ireland, the GB and the EU. | Inbound/outbound RO/RO trade volumes between Ireland and the GB (tonnes). |
|  | Inbound/outbound RO/RO trade volumes between Ireland and the EU (Land-bridge) (tonnes). |
|  | Inbound/Outbound RO/RO trade volumes between Ireland and the EU (Direct Route) (tonnes). |
| Non-tariff barriers at Irish, GB, and EU ports. | Checks on inbound/outbound trucks at Irish ports (Dublin and Rosslare). |
|  | Checks on inbound/outbound trucks at west GB ports (Heysham, Liverpool, Holyhead, Fishguard, and Pembroke). |
|  | Checks on inbound/outbound trucks at east GB port (Dover). |
|  | Checks on inbound/outbound trucks at EU ports. |
| Checking resources capacity. | Number of customs, DAFM, and security officers at Irish ports |
|  | Number of customs, DAFM, and security officers at west GB ports |
|  | Number of customs, DAFM, and security officers at east GB port |
|  | Number of customs, DAFM, and security officers at EU port |
| Vessel average capacity on specific maritime routes. | Proportions of the trucks exported from different Irish ports (%). |
|  | Ferries capacities that carry outbound trucks (freight units). |
|  | Proportions of the trucks imported to Ireland from different west GB ports (%). |
|  | Ferries capacities that carry inbound/outbound trucks through direct route (freight units). |
| Average shelf life. | Average shelf life of specific Agri food product category (days). |
| Transportation Modes. | Percentage of accompanied/unaccompanied trucks at Irish ports (%). |
| Border Checking Cost. | Fees of different checks applied on every truck (€). |
| Measurement Indicators |  |
| Inbound/outbound Transportation Time. | Inbound/outbound transportation time for trucks between Ireland and the GB (hours). |
|  | Inbound/outbound transportation time for trucks flow between Ireland and the EU (Land-bridge) (hours). |
|  | Inbound/outbound transportation time for trucks flow between Ireland and the EU (Direct Route) (hours). |
| Shelf-life Time. | Remaining Shelf-life time for inbound/outbound products between Ireland and GB (%). |
|  | Remaining Shelf-life time for inbound/outbound products between Ireland and the EU (land-bridge) (%). |
|  | Remaining Shelf-life time for inbound/outbound products between Ireland and the EU (Direct Route) (%). |
| Trucks waiting time at the checkpoints. | Inbound/outbound trucks waiting time at the checkpoint in the Irish Ports (hr). |
|  | Inbound/outbound trucks waiting time at the checkpoint in the GB Ports (Eastern and Western ports) (hr). |
|  | Inbound/outbound trucks waiting time at the checkpoint in the EU Ports (hr). |
| Check Resource Utilization. | Customs officers utilization percentage (%). |
|  | DAFM officers’ utilization percentage (%). |
| Trucks Queue Length. | Trucks Queue Length for rolling on the Vessels at Irish Ports. |
|  | Trucks Queue Length for rolling on the Vessels at the GB Ports. |
|  | Trucks Queue Length for rolling on the Vessels at the EU Ports. |

## Simulation-based scenario mapping model

Discrete-event simulation, along with Agent based methods, were utilized to build the simulation-based scenario mapping model. The simulation platform Anylogic was used. This platform has the capability to mimic the road and maritime traffic patterns, together with specific trip times based on a GIS Map, see Figure 6. It is flexible and allows various road and maritime routes to be considered in detail, along with different classes of vehicles.

Figure 6. Anylogic simulation model with GIS map.

In total, 17 agents were used to mimic the flow of import and exported products between Ireland, GB and the EU. The main agent is developed to manage the dynamics of the overall agents and model their interrelationships. The agents include, supplier agents (i.e. the origin point generating the trucks), distributor agents (i.e. the destination point receiving the trucks), and port agents. These model port operations and proposed check procedures. Inbound and Outbound truck flows are modelled as below:

- Outbound trucks flow: The simulation begins by generating the trucks from an Irish-based supplier “supplier agent”. The number of trucks is identified based on the product volumes entered through the model’s control screen (i.e. input parameters). Trucks then head either to Dublin or Rosslare ports according to identified percentage which is fed through the model’s control screen “port agent”. Upon arrival at the ports, trucks will roll-on to the available vessels as per their capacities. Vessels then sail to their destination according to the identified maritime routes. Upon arrival to their destined port, trucks roll-off from the vessels and depart to either GB-based distributor or EU-based distributor “distributor agent” (i.e. any locations at GB or EU-26).

- Inbound trucks flow: Similarly, trucks are generated from EU/GB-based supplier “supplier agent”. Trucks then move to their origin ports as per specific maritime routes “port agents”. According to vessel scheduling, trucks roll/on to the vessels and then sail to their destination ports. Upon arrival at destined port trucks they roll-off from the vessel and eventually end their journeys at Irish-based distributor ”distributor agent” (i.e. any location in Republic of Ireland).

Outbound trucks from Ireland to GB may be subjected to export procedures and security regimes at Irish ports following the end of the Brexit transition period. On the other hand, there will be higher level of checks interventions by customs and SPS agencies to inbound trucks from GB to Ireland. This is based on agencies risk assessment of the submitted declarations. Trucks which move between Ireland and the EU through Direct Route services, will not be checked at any ports. However, trucks which use the Land-bridge to the mainland Europe may be subjected to internal transit checks at the first entry ports to GB.

# Appendix

## List of Input Parameters

The list of input parameters is divided in seven sub sections, as represented below:

| Section1: Ro/Ro annual trade volumes between Ireland (IR), GB and the EU (2018). |  |  |
| --- | --- | --- |
| - Ro/Ro Trade Vol Between IR-GB. |  |  |
| Import Volumes for all products from GB to IRE (tonnes). | 6,330,240 |  |
| Export Volumes for all products from IRE to GB (tonnes). | 5,554,080 |  |
| Import Volumes for Agri-food products from GB to IRE (tonnes). | 1,643,898 |  |
| Export Volumes for Agri-food products from IRE to GB (tonnes). | 1,340,611 |  |
| Import Volumes for Meat category (e.g. Meat) from GB to IRE (tonnes). | 122,903 |  |
| Export Volumes for Meat category (e.g. Meat) from GB to IRE (tonnes). | 454,859 |  |
| Import Volumes for Dairy product category from GB to IRE (tonnes). | 193,505 |  |
| Export Volumes for Dairy product category from GB to IRE (tonnes). | 237,379 |  |
| Import Volumes for Fish product category from GB to IRE (tonnes). | 46,125 |  |
| Export Volumes for Fish product category from GB to IRE (tonnes). | 16,872 |  |
| - Ro/Ro Trade Vol Between IR-EU (via Landbridge). |  |  |
| Import Volumes for all products from EU to IRE through Landbridge (tonnes). | 1,205,760 |  |
| Export Volumes for all products from IRE to EU through Landbridge (tonnes). | 1,057,920 |  |
| Import Volumes for Agri-food products from EU to IRE through Landbridge (tonnes). | 313,123 |  |
| Export Volumes for Agri-food products from IRE to EU through Landbridge (tonnes). | 255,354 |  |
| - Ro/Ro Trade Vol Between IR-EU (via Cherbourg). |  |  |
| Import Volumes for all products from EU (Cherbourg) to IRE through Direct Route (tonnes). | 347,000 |  |
| Export Volumes for all products from IRE to EU (Cherbourg) through Direct Route (tonnes). | 254,000 |  |
| Import Volumes for Agri-food products from EU (Cherbourg) to IRE through Direct Route (tonnes). | 129,149 |  |
| Export Volumes for Agri-food products from IRE to EU (Cherbourg) through Direct Route (tonnes). | 35,282 |  |
| - Ro/Ro Trade Vol Between IR-EU (via Rotterdam). |  |  |
| Import Volumes for all products from EU (Rotterdam) to IRE through Direct Route (tonnes). | 417,000 |  |
| Export Volumes for all products from IRE to EU (Rotterdam) through Direct Route (tonnes). | 256,000 |  |
| Import Volumes for Agri-food products from EU (Rotterdam) to IRE through Direct Route (tonnes). | 153,985 |  |
| Export Volumes for Agri-food products from IRE to EU (Rotterdam) through Direct Route (tonnes). | 35,282 |  |
| - Ro/Ro Trade Vol Between IR-EU (via Zeebrugge). |  |  |
| Import Volumes for all products from EU (Zeebrugge) to IRE through Direct Route (tonnes). | 593,000 |  |
| Export Volumes for all products from IRE to EU (Zeebrugge) through Direct Route (tonnes). | 290,000 |  |
| Import Volumes for Agri-food products from EU (Zeebrugge) to IRE through Direct Route (tonnes). | 218,560 |  |
| Export Volumes for Agri-food products from IRE to EU (Zeebrugge) through Direct Route (tonnes) | 39,692 |  |
| - Ro/Ro Trade Vol Between IR-EU (via Bilbao). |  |  |
| Import Volumes for all products from EU (Bilbao) to IRE through Direct Route (tonnes). | 0 |  |
| Export Volumes for all products from IRE to EU (Bilbao) through Direct Route (tonnes). | 0 |  |
| Import Volumes for Agri-food products from EU (Bilbao) to IRE through Direct Route (tonnes). | 0 |  |
| Export Volumes for Agri-food products from IRE to EU (Bilbao) through Direct Route (tonnes). | 0 |  |
| - Ro/Ro Trade Vol Between GB-EU. |  |  |
| Import Volumes from the EU-26 to GB (tonnes). | 7551000 |  |
| Export Volumes from the GB to the EU-26 (tonnes). | 10853000 |  |
| Section 2: Non-tariff Barriers and Checks at IRE, GB-West, GB-East & EU Ports. |  |  |
| - Checks on Irish-Ports for Non-Agri Products (Exports To GB). |  |  |
| Percentage of outbound trucks subject to customs intervention at Irish Ports (%). | 100% |  |
| Timing of Customs Intervention on outbound trucks at Irish ports (minutes). | 0 |  |
| - Checks on Irish-Ports for Agri-Products (Exports To GB). |  |  |
| Percentage of outbound Agri-food trucks subject to customs intervention at Irish Ports (%). | 100% |  |
| Timing of Customs Intervention on outbound Agri-food trucks at Irish ports (minutes). | 0 |  |
| - Checks on Irish-Ports for Product Category (Exports To GB). |  |  |
| Percentage of outbound Product Category trucks subject to customs intervention at Irish Ports (%). | 100% |  |
| Timing of Customs Intervention on outbound Product Category trucks at Irish ports (minutes). | 0 |  |
| - Checks on Irish-Ports for Non-Agri products (Imports From GB). |  |  |
| Percentage of inbound trucks directed to green route at Irish Ports (%). | 100% |  |
| Timing of documentary and sealed identity checks on inbound trucks at Irish ports (orange routed) (minutes). | 0 |  |
| Percentage of inbound trucks need physical checks (red routed) at Irish Ports (%). | 0% |  |
| Timing of physical checks on inbound trucks at Irish ports (minutes). | 0 |  |
| Percentage of inbound trucks for security, license compliance and immigration checks at Irish Ports (%). | 0% |  |
| Timing of the security, license compliance and immigration checks on inbound trucks at Irish ports (minutes). | 0 |  |
| - Checks on Irish-Ports for Agri-food products (Imports From GB). |  |  |
| Timing of documentary and sealed identity checks on Agri-food inbound trucks at Irish ports (orange routed) (minutes). | 0 |  |
| Percentage of Agri-food inbound trucks need physical checks (red routed) at Irish Ports (%). | 0% |  |
| Timing of physical checks on Agri-food inbound trucks at Irish ports (minutes). | 0 |  |
| Percentage of Agri-food inbound trucks for security, license compliance and immigration checks at Irish Ports (%). | 0% |  |
| Timing of the security, license compliance and immigration checks on Agri-food inbound trucks at Irish ports (minutes). | 0 |  |
| - Checks on Irish-Ports for Product Category products (Imports From GB). |  |  |
| Timing of documentary and sealed identity checks on Product category (e.g. Meat) inbound trucks at Irish ports (orange routed) (minutes). | 0 |  |
| Percentage of Product category (e.g. Meat) inbound trucks need physical checks (red routed) at Irish Ports (%). | 0% |  |
| Timing of physical checks on Product category (e.g. Meat) inbound trucks at Irish ports (minutes). | 0 |  |
| Percentage of Product category (e.g. Meat) inbound trucks for security, license compliance and immigration checks at Irish Ports (%). | 0% |  |
| Timing of the security, license compliance and immigration checks on Product category (e.g. Meat) inbound trucks at Irish ports (minutes). | 0 |  |
| - Checks on West GB Ports for Non-Agri Products (Exports To IRE). |  |  |
| Percentage of outbound trucks subject to customs intervention at West GB Ports (%). | 100% |  |
| Timing of Customs Intervention on outbound trucks at West GB ports (minutes). | 0 |  |
| - Checks on West GB Ports for Agri-food Products (Exports To IRE) |  |  |
| Percentage of outbound Agri-food trucks subject to customs intervention at West GB Ports (%).. | 100% |  |
| Timing of Customs Intervention on outbound Agri-food trucks at West GB ports (minutes). | 0 |  |
| - Checks on West GB Ports for Product category (Exports To IRE). |  |  |
| Percentage of outbound product category trucks subject to customs intervention at West GB Ports (%). | 100% |  |
| Timing of Customs Intervention on outbound product category trucks at West GB ports (minutes). | 0 |  |
| - Checks on West GB Ports for Non-Agri products (Imports From IRE). |  |  |
| Percentage of inbound trucks directed to green route at west GB Ports (%). |  | 100% |
| Timing of documentary and sealed identity checks on inbound trucks at west GB Ports (orange routed) (minutes). |  | 0 |
| Percentage of inbound trucks need physical checks (red routed) at west GB Ports (%). |  | 0% |
| Timing of physical checks on inbound trucks at west GB Ports (minutes). |  | 0 |
| Percentage of inbound trucks for security, license compliance and immigration checks at west GB Ports (%). |  | 0% |
| Timing of the security, license compliance and immigration checks on inbound trucks at west GB Ports (minutes). |  | 0 |
| - Checks on West-GB Ports for Agri-food products (Imports From IRE). |  |  |
| Timing of documentary and sealed identity checks on Agri-food inbound trucks at west GB Ports (Orange routed) (minutes). | 0 |  |
| Percentage of Agri-food inbound trucks need physical checks (red routed) at west GB Ports (%). | 0% |  |
| Timing of physical checks on Agri-food inbound trucks at west GB Ports (minutes). | 0 |  |
| Percentage of Agri-food inbound trucks for security, license compliance and immigration checks at west GB Ports (%). | 0% |  |
| Timing of the security, license compliance and immigration checks on Agri-food inbound trucks at west GB Ports (minutes). | 0 |  |
| - Checks on West-GB ports for Product Category products (Imports From IRE). |  |  |
| Timing of documentary and sealed identity checks on Product category (e.g. Meat) inbound trucks at west GB Ports (Orange Routed) (minutes). | 0 |  |
| Percentage of Product category (e.g. Meat) inbound trucks need physical checks (red routed) at west GB Ports (%). | 0% |  |
| Timing of physical checks on Product category (e.g. Meat) inbound trucks at west GB Ports (minutes). | 0 |  |
| Percentage of Product category (e.g. Meat) inbound trucks for security, license compliance and immigration checks at west GB Ports (%). | 0% |  |
| Timing of the security, license compliance and immigration checks on Product category (e.g. Meat) inbound trucks at west GB Ports (minutes). | 0 |  |
| - Checks on East GB Ports (Dover) for All Products (Exports To EU). |  |  |
| Percentage of outbound trucks subject to customs intervention at East GB Ports (%). | 100% |  |
| Timing of Customs Intervention on outbound trucks at East GB ports (minutes). | 0 |  |
| - Checks on East GB Ports (Dover) for All products (Imports From EU). |  |  |
| Percentage of inbound trucks directed to green route at East GB Port (Dover) (%). | 100% |  |
| Timing of documentary and sealed identity checks on inbound trucks at East GB Port (Dover)(minutes). | 0 |  |
| Percentage of inbound trucks need physical checks (red routed) at East GB Port (Dover) (%). | 0% |  |
| Timing of physical checks on inbound trucks at East GB Port (Dover) (minutes). | 0 |  |
| Percentage of inbound trucks for security, license compliance and immigration check at East GB Port (Dover) (%). | 0% |  |
| Timing of the security, license compliance and immigration checks on inbound trucks at East GB Port (Dover) (minutes). | 0 |  |
| - Checks on West-EU Ports (Calais) for All Products (Exports To the GB). |  |  |
| Percentage of outbound trucks subject to customs intervention at East GB Ports (%). | 100% |  |
| Timing of Customs Intervention on outbound trucks at East GB ports (minutes). | 0 |  |
| - Checks on West-EU Ports (Calais) for All products (Imports From the GB). |  |  |
| Percentage of inbound trucks directed to green route at West-EU Ports (Calais) (%). | 100% |  |
| Timing of documentary and sealed identity checks on inbound trucks at West-EU Ports (Calais) (minutes). | 0 |  |
| Percentage of inbound trucks need physical checks (red routed) at West-EU Ports (Calais) (%). | 0% |  |
| Timing of physical checks on inbound trucks at West-EU Ports (Calais) (minutes). | 0 |  |
| Percentage of inbound trucks for security, license compliance and immigration check at West-EU Ports (Calais) (%). | 0% |  |
| Timing of the security, license compliance and immigration checks on inbound trucks at West-EU Ports (Calais) (minutes). | 0 |  |
| Section 3: Transit checks at GB West and East ports. |  |  |
| Transit check time for all trucks flow between IRE and EU-26 through the GB Landbridge (minutes). | 0 |  |
| Percentage of the trucks selected for physical checks at the GB ports (if borders checks are applied on the trucks using Landbridge) (%). | 0% |  |
| Timing of physical checks for the trucks flow between IRE and EU-26 through Landbridge at the GB ports (minutes). | 0 |  |
| Percentage of the trucks selected for security checks at the GB ports (if borders checks are applied on the trucks using Landbridge) (%). | 0% |  |
| Timing of security, license compliance and immigration checks for the trucks flow between IRE and EU-26 through Landbridge at the GB ports (minutes). | 0 |  |
| Section – 4: Resources Capacity for the check points at IR, GB and EU Ports |  |  |
| - Capacity/Resources at Irish-Ports. |  |  |
| Number of custom officers at Dublin port. | 10 |  |
| Number of DAFM officer at Dublin port. | 10 |  |
| Number of security and document compliance officers at Dublin port. | 10 |  |
| Number of tractors used to tow an accompanied truck at Dublin port. | 20 |  |
| Number of custom officers at Rosslare port. | 10 |  |
| Number of DAFM officer at Rosslare port. | 10 |  |
| Number of security and document compliance officers at Rosslare port. | 10 |  |
| Number of tractors used to tow an accompanied truck at Rosslare port. | 20 |  |
| - Capacity/Resources at GB-West Ports. |  |  |
| Number of custom officers at GB-West ports. | 10 |  |
| Number of DAFM officer at GB-West ports. | 10 |  |
| Number of security and document compliance officers at GB-West ports. | 10 |  |
| Number of tractors used to tow an accompanied truck at GB-West ports. | 20 |  |
| - Capacity/Resources at GB-East Port (Dover). |  |  |
| Number of custom officers at GB-East port. | 10 |  |
| Number of DAFM officer at GB-East port. | 10 |  |
| Number of security and document compliance officers at GB-East port. | 10 |  |
| Number of tractors used to tow an accompanied truck at GB-East port. | 20 |  |
| - Capacity/Resources at EU Port (Calais). |  |  |
| Number of custom officers at EU port. | 10 |  |
| Number of DAFM officer at EU port. | 10 |  |
| Number of security and document compliance officers at EU port. | 10 |  |
| Number of tractors used to tow an accompanied truck at EU port. | 20 |  |
| Section5: Vessel average capacities for the Maritime Routes between IR, GB, and the EU. |  |  |
| - Maritime Routes (Exports To GB). |  |  |
| Percentage of outbound trucks from Dublin port to the GB market (%). | 85% |  |
| Percentage of outbound trucks from Dublin port to the EU-26 market (via Landbridge) (%). | 85% |  |
| Average ferry capacity between Dublin and Heysham ports (trailer). | 122 |  |
| Average ferry capacity between Dublin and Liverpool (trailer). | 123 |  |
| Average ferry capacity between Dublin and Holyhead (Trailer). | 209 |  |
| Average ferry capacity between Rosslare and Fishguard (Trailer). | 75 |  |
| Average ferry capacity between Rosslare and Pembroke (Trailer). | 122 |  |
| - Maritime Routes (Imports from GB). |  |  |
| Percentage of outbound trucks from Heysham to Dublin ports (%). | 3% |  |
| Percentage of outbound trucks from Liverpool to Dublin ports (%). | 40% |  |
| Percentage of outbound trucks from Holyhead to Dublin ports (%). | 47% |  |
| Percentage of outbound trucks from Fishguard to Rosslare ports (%). | 4% |  |
| Percentage of outbound trucks from Pembroke to Rosslare ports (%). | 6% |  |
| Percentage of outbound trucks from Heysham to Dublin ports (%). | 0 |  |
| Percentage of outbound trucks from Liverpool to Dublin ports (%). | 10% |  |
| Percentage of outbound trucks from Holyhead to Dublin ports (%). | 80% |  |
| Percentage of outbound trucks from Fishguard to Rosslare ports (%). | 5% |  |
| Percentage of outbound trucks from Pembroke to Rosslare ports (%). | 5% |  |
| - Maritime Direct Routes (IRE to EU) (EU to IRE). |  |  |
| Average ferry capacity between Dublin and Cherbourg ports (trailer). | 170 |  |
| Average ferry capacity between Dublin and Rotterdam (trailer). | 530 |  |
| Average ferry capacity between Dublin and Zeebrugge (trailer). | 530 |  |
| Average ferry capacity between Rosslare and Cherbourg (trailer). | 150 |  |
| Average ferry capacity between Rosslare and Bilbao (trailer). | 80 |  |
| Section 6: Average shelf life of product category. |  |  |
| Average Shelf-life time for the studied product category (e.g. Meat) (days). | 30 |  |
| Section 7: Type of Trucks. |  |  |
| Percentage of the unaccompanied trucks in Dublin and Rosslare ports (%). | 50% |  |
| Section 8: Different Cost for Checks. |  |  |
| Official documentary check cost at Irish ports (€/truck). | 50 |  |
| Official physical check cost at Irish ports (€/truck). | 500 |  |
| Official security check cost at Irish ports (€/truck). | 500 |  |

## List of Performance Indicators

Section1: Transportation time of the inbound/outbound trucks between IR, GB and the EU.

- Avg Transportation Time for non-agri products trucks flow between Ireland and the UK.

Transportation Time for Non-Agri products outbound trucks from IRE to UK.

Transportation Time for Non-Agri Products inbound trucks from UK to IRE.

- Avg Transportation Time for agri products trucks flow between Ireland and the UK.

Transportation Time for Agri products outbound trucks from IRE to UK.

Transportation Time for Agri Products inbound trucks from UK to IRE.

- Avg Transportation Time for product category trucks flow between Ireland and the UK.

Transportation Time for product category outbound trucks from IRE to UK.

Transportation Time for product category inbound trucks from UK to IRE.

- Avg Transportation Time between Ireland and EU through Landbridge.

Transportation Time for all outbound trucks (Non-Agri, Agri-food, specific product category) from IRE to EU through Landbridge.

Transportation Time for all outbound trucks (Non-Agri, Agri-food, specific product category) from IRE to EU through Landbridge.

- Avg Transportation Time between Ireland and EU through Cherbourg.

Transportation Time for all outbound trucks (Non-Agri, Agri-food, specific product category) from IRE to EU through Cherbourg

Transportation Time for all outbound trucks (Non-Agri, Agri-food, specific product category) from IRE to EU through Cherbourg

- Avg Transportation Time between Ireland and EU through Rotterdam and Zeeburrge.

Transportation Time for all outbound trucks (Non-Agri, Agri-food, specific product category) from IRE to EU through Rotterdam and Zeeburrge.

Transportation Time for all outbound trucks (Non-Agri, Agri-food, specific product category) from IRE to EU through Rotterdam and Zeeburrge.

- Avg Transportation Time between Ireland and EU through Bilbao.

Transportation Time for all outbound trucks (Non-Agri, Agri-food, specific product category) from IRE to EU through Bilbao.

Transportation Time for all outbound trucks (Non-Agri, Agri-food, specific product category) from IRE to EU through Bilbao.

Section 2: Remaining product shelf life (Impact of transportation time).

- Remaining Shelf-life for the product category exported/imported to and from UK market.

Remaining shelf life time for the outbound studied product category exported to the UK.

Remaining shelf life time for the inbound studied product category imported from the UK.

- Remaining Shelf-life for the product category exported/imported to and from the EU market through the Landbridge.

Remaining shelf life time for the outbound studied product category exported to the EU through Landbridge.

Remaining shelf life time for the inbound studied product category imported from the EU through Landbridge.

- Remaining Shelf-life for the product category exported/imported to and from the EU market through Cherbourg.

Remaining shelf life time for the outbound studied product category exported to the EU through Cherbourg

Remaining shelf life time for the inbound studied product category imported from the EU through Cherbourg

- Remaining Shelf-life for the product category exported/imported to and from the EU market through Rotterdam and Zeeburrge.

Remaining shelf life time for the outbound studied product category exported to the EU through Rotterdam and Zeeburrge.

Remaining shelf life time for the inbound studied product category imported from the EU through Rotterdam and Zeeburrge.

- Remaining Shelf-life for the product category exported/imported to and from the EU market through Bilbao.

Remaining shelf life time for the outbound studied product category exported to the EU through Bilbao.

Remaining shelf life time for the inbound studied product category imported from the EU through Bilbao.

Section 3: Trucks waiting time due to non-tariff barriers at IR, GB and the EU ports.

- Non-Agri products outbound trucks waiting time because of Non-tariff barriers at Irish ports.

Non-agri products outbound Trucks avg. waiting time because of the checks at Dublin port.

Non-agri products outbound Trucks avg. waiting time because of the checks at Rosslare port.

- Agri products outbound trucks waiting time because of Non-tariff barriers at Irish ports.

Agri products outbound Trucks avg. waiting time because of the checks at Dublin port

Agri products outbound Trucks avg. waiting time because of the checks at Rosslare port

- Product category outbound trucks waiting time because of Non-tariff barriers at Irish ports.

Product category outbound Trucks avg. waiting time because of the checks at Dublin port.

Product category outbound Trucks avg. waiting time because of the checks at Rosslare port.

- Non-Agri products inbound trucks waiting time because of Non-tariff barriers at Irish ports.

Non-agri products inbound Trucks avg. waiting time because of the checks at Dublin port.

Non-agri products inbound Trucks avg. waiting time because of the checks at Rosslare port.

- Agri products inbound trucks waiting time because of Non-tariff barriers at Irish ports.

Agri products inbound Trucks avg. waiting time because of the checks at Dublin port.

Agri products inbound Trucks avg. waiting time because of the checks at Rosslare port.

- Product category inbound trucks waiting time because of Non-tariff barriers at Irish ports.

Product category inbound Trucks avg. waiting time because of the checks at Dublin port.

Product category inbound Trucks avg. waiting time because of the checks at Rosslare port.

- Non-Agri products outbound trucks waiting time because of Non-tariff barriers at GB-West ports.

Non-agri products outbound Trucks avg. waiting time because of the checks at Heysham port.

Non-agri products outbound Trucks avg. waiting time because of the checks at Liverpool port.

Non-agri products outbound Trucks avg. waiting time because of the checks at Holyhead port.

Non-agri products outbound Trucks avg. waiting time because of the checks at Fishguard port.

Non-agri products outbound Trucks avg. waiting time because of the checks at Pembroke port.

- Agri products outbound trucks waiting time because of Non-tariff barriers at GB-West ports.

Agri products outbound Trucks avg. waiting time because of the checks at Heysham port.

Agri products outbound Trucks avg. waiting time because of the checks at Liverpool port.

Agri products outbound Trucks avg. waiting time because of the checks at Holyhead port.

Agri products outbound Trucks avg. waiting time because of the checks at Fishguard port.

Agri products outbound Trucks avg. waiting time because of the checks at Pembroke port.

- Product category outbound trucks waiting time because of Non-tariff barriers at GB-West ports.

Product category outbound Trucks avg. waiting time because of the checks at Heysham port

Product category outbound Trucks avg. waiting time because of the checks at Liverpool port

Product category outbound Trucks avg. waiting time because of the checks at Holyhead port

Product category outbound Trucks avg. waiting time because of the checks at Fishguard port

Product category outbound Trucks avg. waiting time because of the checks at Pembroke port

- Non-Agri products inbound trucks waiting time because of Non-tariff barriers at GB-West ports.

Non-agri products inbound Trucks avg. waiting time because of the checks at Heysham port.

Non-agri products inbound Trucks avg. waiting time because of the checks at Liverpool port.

Non-agri products inbound Trucks avg. waiting time because of the checks at Holyhead port.

Non-agri products inbound Trucks avg. waiting time because of the checks at Fishguard port.

Non-agri products inbound Trucks avg. waiting time because of the checks at Pembroke port.

- Agri products inbound trucks waiting time because of Non-tariff barriers at GB-West ports.

Agri products inbound Trucks avg. waiting time because of the checks at Heysham port.

Agri products inbound Trucks avg. waiting time because of the checks at Liverpool port.

Agri products inbound Trucks avg. waiting time because of the checks at Holyhead port.

Agri products inbound Trucks avg. waiting time because of the checks at Fishguard port.

Agri products inbound Trucks avg. waiting time because of the checks at Pembroke port.

- Product category inbound trucks waiting time because of Non-tariff barriers at GB-West ports.

Product category inbound Trucks avg. waiting time because of the checks at Heysham port.

Product category inbound Trucks avg. waiting time because of the checks at Liverpool port.

Product category inbound Trucks avg. waiting time because of the checks at Holyhead port.

Product category inbound Trucks avg. waiting time because of the checks at Fishguard port.

Product category inbound Trucks avg. waiting time because of the checks at Pembroke port.

- Non-Agri products outbound trucks waiting time because of Non-tariff barriers at GB-East port (Dover) & EU port (Calais).

Non-agri products outbound Trucks avg. waiting time because of the checks at Dover port.

Non-agri products outbound Trucks avg. waiting time because of the checks at Calais port.

- Agri products outbound trucks waiting time because of Non-tariff barriers at GB-East port (Dover) & EU port (Calais).

Agri products outbound Trucks avg. waiting time because of the checks at Dover port.

Agri products outbound Trucks avg. waiting time because of the checks at Calais port.

- Product category outbound trucks waiting time because of Non-tariff barriers at GB-East port (Dover) & EU port (Calais).

Product category outbound Trucks avg. waiting time because of the checks at Dover port.

Product category outbound Trucks avg. waiting time because of the checks at Calais port.

- Non-Agri products inbound trucks waiting time because of Non-tariff barriers at GB-East port (Dover) & EU port (Calais).

Non-agri products inbound Trucks avg. waiting time because of the checks at Dover port.

Non-agri products inbound Trucks avg. waiting time because of the checks at Calais port.

- Agri products inbound trucks waiting time because of Non-tariff barriers at GB-East port (Dover) & EU port (Calais).

Agri products inbound Trucks avg. waiting time because of the checks at Dover port.

Agri products inbound Trucks avg. waiting time because of the checks at Calais port.

- Product category inbound trucks waiting time because of Non-tariff barriers at GB-East port (Dover) & EU port (Calais).

Product category inbound Trucks avg. waiting time because of the checks at Dover port.

Product category inbound Trucks avg. waiting time because of the checks at Calais port.

Section 4: Resource utilisation at Dublin and Rosslare Port including (Custom officers, DAFM officers and Security officers).

Utilisation of Custom Officers at Dublin Port.

Utilisation of DAFM Officers at Dublin Port.

Utilisation of Tractors at Dublin Port.

Utilisation of Security Officers at Dublin Port.

Utilisation of Custom Officers at Rosslare Port.

Utilisation of DAFM Officers at Rosslare Port.

Utilisation of Tractors at Rosslare Port.

Section 5: Trucks Queue length for Ferries.

Outbound trucks queue length from Dublin to the GB-West ports.

Outbound trucks queue length from Dublin to Cherbourg port.

Outbound trucks queue length from Dublin to Rotterdam port.

Outbound trucks queue length from Dublin to Zeebrugge port.

Outbound trucks queue length from Rosslare to the GB-West ports.

Outbound trucks queue length from Rosslare to Cherbourge port.

Outbound trucks queue length from Rosslare to Bilbao port.

Outbound trucks queue length from Heysham to Dublin port.

Outbound trucks queue length from Liverpool to Dublin port.

Outbound trucks queue length from Holyhead to Dublin port.

Outbound trucks queue length from Fishguard to Rosslare port.

Outbound trucks queue length from Pembroke to Rosslare port.

Outbound trucks queue length from Cherbourg to Dublin port.

Outbound trucks queue length from Rotterdam to Dublin port.

Outbound trucks queue length from Zeebrugge to Dublin port.

Outbound trucks queue length from Bilbao to Rosslare port.

Section 6: Official Check costs at Irish Ports.

Total border checking costs at Dublin port for outbound non-agri trucks.

Total border checking costs at Dublin port for outbound agri trucks.

Total border checking costs at Dublin port for outbound product category trucks.

Total border checking costs at Dublin port for inbound non-agri trucks.

Total border checking costs at Dublin port for inbound agri trucks.

Total border checking costs at Dublin port for inbound product category trucks.

Total border checking costs at Rosslare port for outbound non-agri trucks.

Total border checking costs at Rosslare port for outbound agri trucks.

Total border checking costs at Rosslare port for outbound product category trucks.

Total border checking costs at Rosslare port for inbound non-agri trucks.

Total border checking costs at Rosslare port for inbound agri trucks.

Total border checking costs at Rosslare port for inbound product category trucks.

IMDO, 2019, Irish Maritime Transport Economist, Volume 16.

Statistics of Port Traffic, June 2019 published by Central Statistics Office.

CSO Statistics of Port Traffic 2018.

Irish Maritime Development Office (IMDO) – Implication of Brexit on using Landbridge.

Department of Agricultural, Food, and the Marine: Brexit Preparations, Borders Control.

Irish Maritime Transport Economist 16th Edition 2018.